Analysis of the Linz – Donawitz Process: Production of Steel (98% Purity)

Robinson Bui, Karen Izaguirre, Ezra Ju, Logan Kuhn, Esther Marroquin, Grace Wang

Chemical and Biomolecular Engineering, Johns Hopkins University

EN.540.202: Introduction to Chemical and Biological Process Analysis

Dr. Sourojeet Chakraborty

November 19, 2025

Introduction	3

Background	3

Relevance	4

Problem Statement	4

Relevant Calculations	5

Block Flow Diagram	5

Variable Definitions	5

Mathematical Equations and Relationships	5

First Pass Conversion Scaling	5

Mass Balances	6

Reactor Balances	6

Reacted Materials	8

Purge Stream	9

Recycled Stream	9

Deriving Useful Relationships	9

Pig Iron Mass	9

RR and SS316 Efficiency Relationship	9

Solving Mass Balance	12

Knowns	12

Solving the Prior Equations	12

Energy Balance Background and Major Assumptions	13

Interpretation	14

Process Economy	14

Theoretical Placement	15

References	17

Appendices	19

Appendix A	19

Appendix B	20

Appendix C	21

Analysis of the Linz – Donawitz Process: Production of Steel (98% Purity)

# Introduction

## Background

The Linz-Donawitz (LD) Process is a steelmaking method, now more generally known as the basic oxygen process, that takes a raw input of pig iron and scrap in a blast furnace. These inputs are then blown with pure oxygen oxidizing impurities. Originating in Switzerland by the discovery of Robert Durrer in 1949, the process was named between the two towns where Durrer’s experiment was first used in a factory setting; engineers first utilized this experiment at VÖEST company in Linz, Austria, then another factory in Donawitz, Austria (voestalpine AG, 2013). Durrer’s initial experiments were performed using a “2-tonne experimental converter” inside the battery limits of the VÖEST company plant. Before Durrer’s experimentation with the steelmaking process, there were two other processes that the origins of the LD process is contingent upon; without these initial discoveries the LD process would not be possible.

First was Henry Bessemer’s creation of the Bessemer Process which came about as a necessity for meeting weaponry specifications during the Crimean War. Bessemer had developed an “elongated artillery shell” but also needed a material for the cannons launching these shells to withstand the shell impact, where his work with the puddling furnace and removing carbon from the iron pigs because of the excess oxygen within the furnace itself, purified the iron more so than before. The Bessemer process itself uses air blown from the bottom of a tilting converter, now known as the Bessemer Converter, had the benefits of more heated as well as higher purity steel products; phosphorus-free iron being required to get the results Bessemer was achieving (Saville, 1999).

Secondly, the Linde-Fränkl Process which is a process focused on air separation. More specifically a liquefaction of air compressing, cooling, then expanding, yielding high amounts of oxygen (O2) product. The process was first carried out by Carl von Linde in 1895, and in 1928 the Linde company, which he founded, performed the first official Linde-Fränkl Process (Linde plc, 2019). Optimizations to the process made the accessibility to large amounts of high purity oxygen commercially viable to be used for the step in the LD process where oxygen is blown on the pig iron.

## Relevance

Stainless steel (SS 316) is the product this report will be focusing on and is one of the possible grades of steel produced by the LD Process. The applications for this grade of steel span across various fields, most used for structural reasons such as railings, platforms, and tanks. Pipes, fittings, and even implants are other uses other than the use case of buildings or storage. Examples of the industries stainless steel is used in are the chemical & pharmaceutical, maritime, medical, food & beverage, and automotive & aerospace industries, to name a few. Ultimately, these fields choose SS 316 for its ability to withstand temperatures from 425-860 degrees Celsius for extended periods of time and resistance to corrosion; SS 316 is durable (AMC, 2025).

The origins of the product are recent compared to other well-known materials, only being discovered in 1913. Harry Brearly is attributed to being the founder of the first official stainless steel in Sheffield, United Kingdom for surgical scalpels, tools, and cutlery (Thomas, 2022). However, steel’s use as a durable material for tools has been exploited far before the creation of stainless steel. Though there is no definite answer to the first use of steel, there is ongoing research proving that the alloy appears in different regions in increasingly early time periods.

The following cases are of steel generally, not a particular grade of steel. Most recently, researchers at the University of Freiburg took samples of stelae on rock artifacts from the Iberian Peninsula dating back to 800 BCE and replicated the conditions in which these stelae could be imprinted on the specific rock samples. Their findings proved that only steel tipped chisels would have been able to make these marks (Gonzalez, 2023). India and China are also frequently mentioned in the early history of steel use both for a recurring purpose of the material: weaponry. In the 300 BCE, Wootz steel was a significant part of the trade coming from India; Wootz steel is known for being the basis of Damascus steel, prominent material used in crafting blades amongst many regions and cultures (Srinivasan). China during the Song Dynasty in 1000 CE, known as one of the big economic booms comparing the various dynasty throughout Chinese history, steel was found in many arrowhead tips and other weaponry during the time (Columbia).

## Problem Statement

The LD Process is a batch reaction process that takes pig iron and oxygen and turns it into SS 316 (Stainless Steel – Grade 316, composition: 17% Chromium, 11% Nickel, 2% Molybdenum, 70% Iron, 0.07% Carbon). These reactants will react in a blast furnace / basic oxygen furnace, where oxygen is injected into the furnace, with the heat of reaction of -24.7 kJ/mol (meaning the reaction is exothermic).

The targeted production of SS 316 is 1000kg. Pig iron (4.05% Carbon, 0.5% Silicon, 0.05% Phosphorus, 94.56% Iron, 0.04% Sulfur, 0.8% Manganese) and oxygen are fed into a blast furnace at 8-12 bars, where the reaction reaches a temperature of 1400°C. The furnace is jacketed with molten salts.

The excess reactants and products are then put into a separator where SS 316 is removed from the stream, and the excess reagents are put back into the blast furnace with a recycle ratio of 3. Find the atom efficiency of this process with respect to SS 316 and identify the limiting/excess reagents. Then, find the heat of reaction in an adiabatic process.

# Relevant Calculations

## Block Flow Diagram

### Variable Definitions

For this problem, because all information in the literature has been attained in mass measurements, a mass basis is used to solve. Each capital letter represents the total mass amount of the respective stream, while the letter subscripted with one of the following variables refers to mass of that species within the stream. Likewise, the lowercase and subscripted variable refers to the mass percentage within the compound. In the case of pig iron specifically, when referring to an internal variable, the variable will be in the subscript and the stream will be found in the superscript.

## Mathematical Equations and Relationships

### First Pass Conversion Scaling

kApiAF=piAS

#### Solving for Pig Iron Scaling Constants

It should be noted to the reader that while values from literature were given in percentages, because both piFand pisare given as such, it is suitable to assume the percentage terms cancel out.

4kC=0.1

0.046kSu=0.025

0.9kSi=0

kMn=0.12

0.07kP=0.017

93.984kFe=99.738

kC=0.025

kSu=0.543478261

kSi=0

kMn=0.12

kP=0.242857143

kFe=1.061223187

### Mass Balances

PF+MF+R=F

F = S

S = R + SS + SG + PG

### Reactor Balances

#### Pig Iron

Below, the general equation for the input stream is found with a detailed breakdown of every species in the appendix. Notice that every species, except for oxygen, is recycled to some capacity. Also notice here the 1piiS term. What this does is provide a corrected value, as the nature of extrapolation inherently yields some error. The way this has been corrected for here is by dividing by the total sum of mass.

FA=FA+RA ; A: Every species fed into reactor

PIA=PIiA

PIAF=FA

piAF=PIAFPIF=PIAFPIPF+PIR=PIAPF+PIARPIPF+PIR

PIAFPIPF+PIR;A: every species present in pig iron

kApiAF=piAS∑piiS  ;A: every species present in pig iron

FFe=PIFeF=SFe=PIFeS

PIS*piAS=PIAS

### Reacted Materials

Note to the reader that the first two equations only refer to the species not present in the pig iron. The reason Iron and Carbon still have reaction efficiencies despite already being previously reacted is that they are both required for the formation of steel. For this reason, Carbon and Iron have two efficiencies, that of the Iron refinement referenced earlier, and that of the actual production of steel.

Arctd.=EA*FA

Arctd.+Aunrc.=FA

Ferctd.=EFe*SFe=EFe*FFe

Crctd.=EC*SC=EC*PICS

Crctd.+Cunrc.=PICS

#### Slag Reacted Materials

SGa=PIAF-PIAS ;A=Impurity species within pig iron

SGO=Orctd.

SGL=Lrctd.

#### SS316 Reacted Materials

Finally, discussing our desired product, SS 316, the ultimate step is the easiest. All that must be done is adding the reacted components to find the total mass and divide by that value to find mass percentages.

SS=Crctd.+Ferctd.+Nirctd.+Crrctd.+Morctd.

### Purge Stream

This purge stream will consist of unreacted oxygen, carbon monoxide, and inert material as we are unable to/disinclined from recycling them.

### Recycled Stream

For this analysis, the recycled stream is relatively simple and straightforward. We simply define the recycled stream variables to be equal to the unreacted material we have just calculated save oxygen.

RA=Aunrc.

R=Aunrc.i

RR=RMF+PF

R=RR*F1+RR

## Deriving Useful Relationships

### Pig Iron Mass

PIS=PIFeSpiFeS=FFepiFeS

piFeS=kFepiFeF*piiS

PIS=FFekFepiFeF*piiS

### RR and SS316 Efficiency Relationship

First the following is assumed:

EC≈EFe≈EMo≈ENi≈ECr≈E

This is not entirely necessary, but it assists with the math and can easily be compensated for later.

Arctd.=E*FA

FA-Aunrc.=E*FA

Aunrc.=FA1-E

R=Aunrc.i=Fi1-E+PIunrc.i

PIunrc.i=PISiS+PIPS+PISuS+PIMnS

R=Fi1-E+PISiS+PIPS+PISuS+PIMnS

PIAS=PIS*piAS=FFekFepiFeF*piiS*kApiAF*piiS=kApiAFFFekFepiFeF

=kApiAFFFePIFkFePIFeF=kApiAFkFePIF=PIAFPIFkAkFePIF=kAPIAFkFe=kAFAkFe

R=unrc.Fi1-E+kSiFSikFe+kPFPkFe+kSuFSukFe+kMnFMnkFe

=unrc.Fi1-E+1kFeSi,P,Su,MnkiFi

kunrc.=kFe1-E

Now we are able to simplify and combine terms as follows. A note here for the reader is to bear in mind that the recycled stream summation excludes Oxygen, whereas the Charge Feed (CF) does not. This means we will assume the value of k for Inerts and Oxygen to be 0 with respect to the reaction stream. Carbon will have a combined k value which represents the species' two-fold depreciation with two separate constants.

R=kFekFeunrc.Fi1-E+1kFeSi,P,Su,MnkiFi

=1kFeunrc.kFeFi1-E+1kFeSi,P,Su,MnkiFi=1kFetotkiFi

RR=RMF+PF=kiFikFeMF+PF

PF+MF+R=F

CF = PF+MF

FA=RA+CFA

RR=kiFikFeF-R=kiFikFeCF

=kiRi+CFikFekFeCF

Fi=Ri+CFi

Ri=kikFeFi

kFekiRi=Fi

kFekiRi=Ri+CFi

Ri=CFikFeki-1

RR=kiCFikFeki-1+CFikFeCF=kiCFikFekFe-kikFeCF=kiCFikFe-kiCF

RR=kiCFikFe-kiCF

This relationship proves to us that the the Recycled ratio is dependent upon individual reaction efficiencies, and thus will change directly accordingly to how efficient the steel production is, as well as the mass of our input stream.

## Solving Mass Balance

### Knowns

SS=1000

ssFe=0.69220

ssMo=0.02067

ssNi=0.10724

ssCr=0.1793

ssC=0.0006

### Solving the Prior Equations

Next we assume steady state to say that the mass flow rate of each individual species in the system is equal to the mass flow rate of the species out. If this were not the case, we would have build-up and the system would inherently become unsteady state. We will introduce a new variable, TO, which represents total output. Extensive steps to solve the block flow diagram can be seen in the attached spreadsheet. The spreadsheet functions as a way to numerically calculate values based on varying input parameters. Below will be an example of back-calculations for one species to show how the calculations were physically done. The values seen above are based on current calibrations, and can be altered directly if needed with the exception of carbon as it is contingent upon the pig iron content, and dictated by iron's percentage.

C:

SSC=SS*ssC=5957532g

SSC=SC-Cunrc.

SC=piCSpiiS=0.025fCpiiS*PIS=0.025*0.03481.0126*816.2121=0.701

piCS=0.0008695 (uncorrected)

piCS=0.0025piCF

piCF=0.0348

PIFpiCF=PICF

PIF=855.438

PICF=29.7519

RC+PFC=FC

PFC=FC-RC=PICF-SC-SSC=29.7519-0.701-0.5957532=29.6467

It is seen in the figure above that this value does corroborate the parameters in the spreadsheet and overall mass balance.

# Energy Balance Background and Major Assumptions

Using the mass-balance results at 85% first-pass conversion (FPC), a detailed energy balance was completed for the Linz–Donawitz process. For the energy balance, the fresh feed is assumed to enter the furnace at standard temperature and pressure (25 °C, 1 atm). This feed mixes with the recycle stream and enters the reactor at an increased temperature. The combined stream is heated to 1700 °C and subsequently directed to a separator, where it is divided into product, purge, and recycle streams. All transport lines are assumed to be insulated, resulting in negligible heat losses; therefore, all outlet streams are taken to be at 1700 °C. Kinetic and potential energy effects, as well as shaft work, are neglected due to a lack of available data. Atmospheric effects are also ignored despite gas generation, which simplifies the energy balance such that the total enthalpy change consists of outlet enthalpy, latent heat of melting, heat of reaction, and inlet enthalpy contributions.

Because the inlet temperature to the reactor is not at STP, the temperature which the reactants enter will be the reference temperature. Much like in the mass balance, any unaccounted-for alloy species (e.g., Ni, Cr, Mo) are treated as inert species and considered insignificant for the energy balance. It is further assumed that all meltable species undergo complete melting at the reactor temperature of 1700 °C. In tandem, phase change data will not be accounted for to simplify the calculations.

Enthalpy values for each stream were calculated using specific heat equations. Relevant constants were predominantly found from the NIST WebBook and various supplementary sources and tables. Specific heat data were taken from 298°K to 1973°K; values outside this range were extrapolated to approximate their behavior at higher temperatures.

For the inlet stream, the fresh feed at STP is assigned zero enthalpy. The enthalpy of the recycle stream was calculated using specific heat integration between 25 °C and 1700 °C. Latent heat of fusion within the recycle were neglected because melting in the reactor and subsequent solidification upon mixing with the inlet stream cancel out energetically under the assumption of equal magnitudes. Combining the two, the inlet mixture temperature was calculated to be 324.175 °K (51.025 °C) using numerical methods.

The outlet-stream enthalpy was calculated using specific heat integration from the inlet temperature (51°C) to the outlet temperature (1700°C). Setting the inlet mixture temperature as the reference state ensures the inlet stream has zero enthalpy. Latent heat of melting was added using values obtained from the NIST WebBook. The heat of reaction was calculated using the extent of impurity oxidation determined from the FPC, multiplying by the difference between product and reactant heats of formation. Totaling all the streams, the calculated value of the reaction process is 3.98*105 kJ per kg of SS316 produced. Scaling to a production rate of 1000 kg per day, this results in 3.98*105 MJ per day, or approximately 400 GJ/day.

# Interpretation

Although the reactions within the LD process are exothermic, the overall energy requirement for the process remains extremely endothermic. This is attributed to the intense energy requirement needed to heat metallic species to their melting points and to reform their crystal structure for alloy formation. The calculated energy demand would decrease if the recycle ratio were increased, allowing more thermal energy to be recovered and improve process efficiency.

# Process Economy

The LD process is economically favorable when raw-material cost and product values are considered together. Because the reaction is exothermic (once it has been running for a while), furnace energy requirements are low, meaning most operating costs come from pig iron, alloy additions, and oxygen. Regions with abundant pig iron, like South Africa, benefit from the significant lower pig iron and alloy cost, reducing the overall price per batch of SS 316.

A key economic advantage is the ability to sell slag as a byproduct. In South Africa, slag has strong demand from cement manufacturers, providing a reliable credit that helps with the process cost. Local buyers reduce transportation expenses and ensure consistent offtake.

Revenue potential for SS 316 is high because it is a steel that is used in automotive, maritime, railroads, and manufacturing sectors. South Africa’s established trade links with major industrial economies support stable pricing and easy exports.

Taken together, low domestic material costs, high lag for resale value, and strong demand for SS 316 make the LD process economically viable and competitive in regions with mining and export infrastructure like South Africa.

# Theoretical Placement

After weighing multiple factors such as raw material cost, selling price of the product, and access to transportation, a potential location for this plant is the Coega Special Economic Zone (SEZ) in South Africa. As seen in the process economy analysis, in South Africa, the cost of raw materials needed for the LD Process are low, and the selling price of both SS 316 and slag are high. In 2023, South Africa was the world’s seventh-largest producer of iron ore, the precursor to pig iron (“Iron ore production in South Africa and major projects”). Having domestically produced raw materials reduces transportation costs and the overall cost of the raw materials. In addition, South Africa has strong trading ties to China, the United States, and Germany (“South Africa exports by country”). These countries all have large automotive industries, where SS316 is often used for components like exhaust systems that benefit from the heat resistance and durability of the material (AMC).

Within South Africa, SEZs are geographically designated areas that have different economic policies and special arrangements to support specifically targeted economic activities. Benefits of operating within an SEZ include a reduced rate of corporate income taxation, tax reliefs on buildings, incentives for employment, and customs controlled areas ("Benefits of operating within special economic zones”). Thus, placing the plant within an SEZ will draw in investors and reduce the cost of certain operations. Moreover, Coega is home to companies that could be potential clients. For example, Hella, located in Zone 2 of Coega, produces lighting and electronics for automotives. Hella Marine, a branch of the company, specializes in maritime lighting. Since these products must withstand saltwater environments, they often contain SS 316, which has excellent corrosion resistance. Another client recommendation is Cemza Cement, which uses slag in multiple products (“Cemza Cement Products”). This proximity to potential clients allows for immediate partnerships to be formed and greatly lowers transportation costs. Beyond neighboring companies, Coega is the only SEZ with two seaports—Port Elizabeth and Ngqura—making it an ideal location for trade (“SEZ Investments”). Thus, placing the plant at Coega SEZ provides economic benefits and better opportunity for trade.

# References

AMC. (2025, March 5). What is stainless steel 316?: Uses, grades, and composition. What is Stainless Steel 316?: Uses, Grades, and Composition. https://www.amardeepsteel.com/blog/what-is-stainless-steel-316-uses-grades-and-composition.html

Araque Gonzalez, R., Asmus, B., Baptista, P., Mataloto, R., Paniego Díaz, P., Rammelkammer, V., Richter, A., Vintrici, G., & Rafael Ferreiro Mählmann. (2023). Stone-working and the earliest steel in Iberia: Scientific analyses and experimental replications of Final bronze age stelae and Tools. Journal of Archaeological Science, 152, 105742. https://doi.org/10.1016/j.jas.2023.105742

Asia for Educators, C. U. (n.d.). China in 1000 CE. Song Dynasty China | Asia for Educators. https://afe.easia.columbia.edu/songdynasty-module/econ-rev-iron-steel.html

Benefits of operating within special economic zones (2025). investSA. https://www.investsa.gov.za/special-economic-zones/

Burgess, D. “Thermochemical Data” in NIST Chemistry WebBook, NIST Standard Reference

Database Number 69, Eds. P.J. Linstrom and W.G. Mallard, National Institute of Standards and Technology, Gaithersburg MD, 20899, https://doi.org/10.18434/T4D303, (retrieved November 24, 2025).

Cemza Cement Products (2025). Cemza Engineered Cement. https://cemza.co/cement-products/

Chase, M. W., Davies, C. A., Downey, J. R., Frurip, D. J., McDonald, R. A., & Syverud, A. N.

(1985). NIST-Janaf thermochemical tables. NIST-JANAF Thermochemical Tables. https://janaf.nist.gov/pdf/JANAF-FourthEd-1998-Iron.pdf

Domalski, E., Hearing E., " Condensed Phase Heat Capacity Data" in NIST Chemistry WebBook,

NIST Standard Reference Database Number 69, Eds. P.J. Linstrom and W.G. Mallard, National Institute of Standards and Technology, Gaithersburg MD, 20899, https://doi.org/10.18434/T4D303, (retrieved November 24, 2025).

Iron ore production in South Africa and major projects (2024, Aug 23). Mining Technology. https://www.mining-technology.com/data-insights/iron-ore-in-south-africa/?cf-view

Linde plc. (2019). Air Separation Plants History and technological progress in the course of Time. https://assets.linde.com/-/media/global/engineering/engineering/home/products-and-services/process-plants/air-separation-plants/air-separation-plants-history-and-technological-progress-2019.pdf

Lorig, C. H. (1998, October 30). Ferrous metals. Britannica. https://www.britannica.com/science/metallurgy/Light-metals

Saville, J. P. (1999, May 4). Henry Bessemer | Biography, inventions, & facts | britannica. Henry Bessemer. https://www.britannica.com/biography/Henry-Bessemer

SEZ Investments (2025). COEGA. https://www.coega.co.za/site/sez-investments

South Africa Exports By Country (2025). https://tradingeconomics.com/south-africa/exports-by-country.

Srinivasan, S. (n.d.). WOOTZ STEEL: AN ADVANCED MATERIAL OF THE ANCIENT WORLD. Wootz Steel: An advanced material of the Ancient World. https://dtrinkle.matse.illinois.edu/MatSE584/articles/wootz_advanced_material/wootz_steel.html

Redmond, R. F., & Lones, J., Enthalpies and Heat Capacities of Stainless Steel (316), Zirconium,

and Lithium at Elevated Temperatures (1952). Oak Ridge, Tennessee; Carbide and Carbon Chemical Company. Retrieved 2025, from https://www.osti.gov/servlets/purl/4392113.

Thomas, G. P. (2022a, September 24). The history of Stainless Steel – celebrating 100 years. AZoM. https://www.azom.com/article.aspx?ArticleID=8307

Voestalpine AG. (2013, January). The story of the Linz-Donawitz process A development which has changed the world. https://www.voestalpine.com/group/static/sites/group/.downloads/en/press/2012-broschuere-the-linz-donawitz-process.pdf

# Appendices

## Appendix A

Mass Balance Calculations

Variable/Symbol Names

Reactor Feed

FO=MFO

FC=PFC+RC

FSi=PFSi+RSi

FP=PFP+RP

FFe=PFFe+RFe

FSu=PFSu+RSu

FMn=PFMn+RMn

FCr=MFCr+RCr

FNi=MFNi+RNi

FMo=MFMo+RMo

FL=MFL+RL

Fresh pig iron percentage in feed stream for each species

piCF=PICFPIPF+PIR

piSuF=PISuFPIPF+PIR

piSiF=PISiFPIPF+PIR

piMnF=PIMnFPIPF+PIR

piPF=PIPFPIPF+PIR

piFeF=PIFeFPIPF+PIR

Scaling constant of pig iron species

kCpiCF=piCSpiiS

kSupiSuF=piSuSpiiS

kSipiSiF=piSiSpiiS

kMnpiMnF=piMnSpiiS

kPpiPF=piPSpiiS

kFepiFeF=piFeSpiiS

Slag formation

SGC=PICF-PICS

SGSi=PISiF-PISiS

SGSu=PISuF-PISuS

SGP=PIPF-PIPS

SGMn=PIMnF-PIMnS

## Appendix B

Energy Balance Calculations

Energy balance equation

ΔH+ΔEk+ΔEP=Q-WS

ΔH=outniHi+meltniHfl+rxnξHrxn-inniHi=Q

Calculating inlet temperature

ΔHin=Qrecycled

Q_{recycled}=(ai103T+bi2*106T2+ci3*109T3+di4*1012T4-ei*103T25°C1700°C)ni

ΔHin=-Q+T25TinCpTdT=a103T+b2*106T2+c3*109T3+d4*1012T4-e*103T25°CTin-Q

where a,b,c,d,e=a,b,c,d,ei* ni

Calculating outlet enthalpy

ΔHout=TinToutCpTdT= =(ai103T+bi2*106T2+ci3*109T3+di4*1012T4-ei*103T25°C1700°C)ni

where a,b,c,d,e=a,b,c,d,ei* ni

Calculating heat of melting

Hmelt=meltniHfl

Calculating heat of reaction

Hrxn=ξ Hrxn=ξprodvHf-reactvHf

Table of constants used

## Appendix C

Atom & Process Economy Calculations

Total Mass Atom Economy: 90.82%

| Species | Mass | Mol. Mass | Moles |
| --- | --- | --- | --- |
| Pig Iron: | 731.9358096 | 15.1194157 | 15.1194157 |
| Lime: | 1.056078811 | 56.077 | 0.018832655 |
| Cr: | 180.7583249 | 51.996 | 3.476389048 |
| Ni: | 112.8853595 | 58.693 | 1.923318956 |
| Mo: | 20.69439233 | 95.95 | 0.21567892 |
| O2: | 56.61296979 | 31.998 | 1.769265885 |
| Total: | 1103.942935 | 49.01424231 | 22.52290116 |

Process Economy:

| Species | Price (USD) | Per Unit | Per Unit (g) | Mass Req | Cost | Upscaled | Upsc./year |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Pig Iron: | $300.00 | ton | $0.0003000 | 731.93581 | $0.22 | $219.58 | $80,146.97 |
| Lime: | $20.00 | ton | $0.0000200 | 1.05607881 | $0.00 | $0.02 | $7.71 |
| Cr: | $10.00 | kg | $0.0100000 | 180.758325 | $1.81 | $1,807.58 | $659,767.89 |
| Ni: | $20.00 | kg | $0.0200000 | 112.885359 | $2.26 | $2,257.71 | $824,063.12 |
| Mo: | $78.00 | kg | $0.0780000 | 20.6943923 | $1.61 | $1,614.16 | $589,169.35 |
| O2: | $0.00 | g | $0.0000000 | 56.6129698 | $0.00 | $0.00 | $0.00 |
| Totals: |  |  | $0.1083200 |  | $5.90 | $5,899.05 | $2,153,155.04 |
| Products: |  |  |  | Produced | Revenue |  | $0.00 |
| SS316: | $6,000.00 | ton | $0.0060000 | 1000 | $6.00 | $6,000.00 | $2,190,000.00 |
| Slag: | $60.00 | ton | $0.0000600 | 83.1475951 | $0.00 | $4.99 | $1,820.93 |
| Totals: |  |  | $0.0060600 |  | $6.00 | $6,004.99 | $2,191,820.93 |

Yearly Profit: $38,655.89